# Klair Dashboard

Software Setup Guide

From Blank SD Card to Working Dashboard

Version 1.0 | December 2025

# Contents

1. Prerequisites

2. Flash Raspberry Pi OS

3. First Boot Configuration

4. System Updates & Dependencies

5. Install Node.js

6. Install Python Hardware Libraries

7. Configure GPIO & UART

8. Clone & Setup Dashboard Project

9. Configure Kiosk Mode (Auto-Start)

10. Display Configuration

11. Final Checklist

Appendix A: Useful Commands

Appendix B: Troubleshooting

# 1. Prerequisites

## What You Need

- A computer (Mac, Windows, or Linux) to flash the SD card

- MicroSD card (32GB or larger recommended)

- MicroSD card reader

- WiFi network name and password

- Raspberry Pi Imager software (free download)

## Download Raspberry Pi Imager

Download from: https://www.raspberrypi.com/software/

Available for macOS, Windows, and Ubuntu.

# 2. Flash Raspberry Pi OS

- Insert your MicroSD card into your computer's card reader.

- Open Raspberry Pi Imager.

- Click 'Choose Device' → Select 'Raspberry Pi 4'.

- Click 'Choose OS' → Select 'Raspberry Pi OS (64-bit)' under 'Raspberry Pi OS (other)'.

ℹ️ Why 64-bit? Better performance with Node.js and modern software. Your Pi 4 fully supports it.

- Click 'Choose Storage' → Select your MicroSD card.

- Click the gear icon (⚙️) or 'Edit Settings' to open advanced options.

## Configure Advanced Options (Important!)

In the OS Customisation menu, configure the following:

### General Tab

| Setting | Value |
| --- | --- |
| Set hostname | klair-dashboard |
| Set username | parminder (or your preferred username) |
| Set password | Your secure password |
| Configure WiFi | Enter your WiFi SSID and password |
| WiFi country | GB (United Kingdom) |
| Set locale | Timezone: Europe/London, Keyboard: gb |

### Services Tab

- Enable SSH: ✅ Check this box

- Use password authentication: ✅ Select this option

✓ Tip: Enabling SSH lets you configure the Pi remotely from your MacBook without needing a separate keyboard/mouse.

- Click 'Save' to save your settings.

- Click 'Write' and confirm. This will erase the SD card and write the OS.

- Wait for the write and verification to complete (5-15 minutes).

- Eject the SD card safely when prompted.

# 3. First Boot Configuration

- Insert the MicroSD card into your Raspberry Pi 4.

- Connect the display (if assembled) or an HDMI monitor.

- Connect power to the Raspberry Pi.

- Wait 2-3 minutes for first boot to complete.

## Connect via SSH (Recommended)

From your MacBook terminal:

ssh parminder@klair-dashboard.local

If hostname doesn't resolve, find the IP from your router or use:

ping klair-dashboard.local

Then connect with:

ssh parminder@<IP_ADDRESS>

ℹ️ First connection: You'll be asked to verify the host fingerprint. Type 'yes' to continue.

# 4. System Updates & Dependencies

Once connected via SSH, run the following commands:

## Update the System

sudo apt update && sudo apt upgrade -y

This may take 5-10 minutes. Let it complete fully.

## Install Essential Packages

sudo apt install -y git curl wget build-essential

## Install Chromium (for Kiosk Mode)

sudo apt install -y chromium-browser unclutter

unclutter hides the mouse cursor when not in use—perfect for a dashboard display.

# 5. Install Node.js

We'll use NodeSource to install Node.js 20 LTS (recommended for Next.js).

## Install Node.js 20

# Download and run NodeSource setup script

curl -fsSL https://deb.nodesource.com/setup_20.x | sudo -E bash -

# Install Node.js

sudo apt install -y nodejs

## Verify Installation

node --version   # Should show v20.x.x
npm --version    # Should show 10.x.x

## Install pnpm (Optional but Recommended)

sudo npm install -g pnpm

# 6. Install Python Hardware Libraries

Python is pre-installed on Raspberry Pi OS. We need to add libraries for GPIO and NeoPixel control.

## Install pip (if not present)

sudo apt install -y python3-pip python3-venv

## Install GPIO Library

sudo apt install -y python3-rpi.gpio

## Install NeoPixel Libraries

sudo pip3 install rpi_ws281x adafruit-circuitpython-neopixel --break-system-packages

## Install Flask (for Hardware API)

sudo pip3 install flask flask-cors --break-system-packages

⚠️ Note: The --break-system-packages flag is required on Raspberry Pi OS Bookworm. It's safe for these hardware libraries.

# 7. Configure GPIO & UART

## Enable UART (Optional - for sensor configuration)

If you want to configure the mmWave sensor's sensitivity via UART:

sudo raspi-config

Navigate to:

- Interface Options → Serial Port

- Would you like a login shell over serial? → No

- Would you like serial port hardware enabled? → Yes

- Finish and reboot when prompted

## Add User to GPIO Group

sudo usermod -aG gpio $USER

Log out and back in for this to take effect.

# 8. Clone & Setup Dashboard Project

## Create Project Directory

mkdir -p ~/projects
cd ~/projects

## Clone the Dashboard Repository

git clone https://github.com/YOUR_USERNAME/klair-dashboard.git
cd klair-dashboard

ℹ️ Note: Replace with your actual repository URL once the dashboard is developed and pushed to GitHub.

## Install Dependencies

npm install
# or if using pnpm:
pnpm install

## Configure Environment Variables

Create a .env.local file with your API keys:

cp .env.example .env.local
nano .env.local

Fill in your API keys:

OPENWEATHERMAP_API_KEY=your_key_here
GOOGLE_CLIENT_ID=your_client_id
GOOGLE_CLIENT_SECRET=your_client_secret
JELLYFIN_URL=http://100.103.162.79:8096
JELLYFIN_API_KEY=your_jellyfin_key
HOME_ASSISTANT_URL=http://100.103.162.79:8123
HOME_ASSISTANT_TOKEN=your_ha_token

## Build the Production Version

npm run build

## Test the Dashboard

npm run start

Open a browser and visit http://localhost:3000 to verify it works.

# 9. Configure Kiosk Mode (Auto-Start)

Kiosk mode makes the Pi boot directly into the dashboard in full-screen, with no desktop visible.

## Step 1: Create Dashboard Service

Create a systemd service to run the Next.js server:

sudo nano /etc/systemd/system/klair-dashboard.service

Paste the following content:

[Unit]
Description=Klair Dashboard
After=network.target
[Service]
Type=simple
User=parminder
WorkingDirectory=/home/parminder/projects/klair-dashboard
ExecStart=/usr/bin/npm run start
Restart=on-failure
RestartSec=10
Environment=NODE_ENV=production
Environment=PORT=3000
[Install]
WantedBy=multi-user.target

Enable and start the service:

sudo systemctl enable klair-dashboard
sudo systemctl start klair-dashboard

## Step 2: Create Hardware Sidecar Service

Create a service for the Python hardware controller:

sudo nano /etc/systemd/system/klair-hardware.service

Paste the following:

[Unit]
Description=Klair Hardware Controller
After=network.target
[Service]
Type=simple
User=root
WorkingDirectory=/home/parminder/projects/klair-dashboard/hardware
ExecStart=/usr/bin/python3 server.py
Restart=on-failure
RestartSec=10
[Install]
WantedBy=multi-user.target

Enable and start:

sudo systemctl enable klair-hardware
sudo systemctl start klair-hardware

⚠️ Note: The hardware service runs as root because NeoPixel requires direct hardware access.

## Step 3: Configure Auto-Start Browser (Kiosk)

Create an autostart script for Chromium in kiosk mode:

mkdir -p ~/.config/autostart
nano ~/.config/autostart/kiosk.desktop

Paste the following:

[Desktop Entry]
Type=Application
Name=Klair Dashboard Kiosk
Exec=/home/parminder/kiosk.sh
X-GNOME-Autostart-enabled=true

Create the kiosk launch script:

nano ~/kiosk.sh

Paste:

#!/bin/bash
# Wait for dashboard server to be ready
sleep 10
# Disable screen blanking
xset s off
xset -dpms
xset s noblank
# Hide cursor when idle
unclutter -idle 0.5 -root &
# Launch Chromium in kiosk mode
chromium-browser \
  --kiosk \
  --noerrdialogs \
  --disable-infobars \
  --disable-session-crashed-bubble \
  --disable-restore-session-state \
  --no-first-run \
  --start-fullscreen \
  --autoplay-policy=no-user-gesture-required \
  http://localhost:3000

Make it executable:

chmod +x ~/kiosk.sh

## Step 4: Disable Screen Blanking Permanently

sudo nano /etc/lightdm/lightdm.conf

Find or add under [Seat:*]:

xserver-command=X -s 0 -dpms

# 10. Display Configuration

## Touch Display 2 Settings

The Raspberry Pi Touch Display 2 should work out of the box. If you need to adjust settings:

sudo nano /boot/firmware/config.txt

### Useful Display Settings

| Setting | Effect |
| --- | --- |
| lcd_rotate=2 | Rotate display 180° (if upside down) |
| display_lcd_rotate=2 | Alternative rotation method |
| disable_splash=1 | Disable rainbow splash on boot |

## Adjust Brightness (Software)

The Touch Display 2 supports software brightness control:

# Set brightness (0-255)
echo 128 | sudo tee /sys/class/backlight/*/brightness
# Read current brightness
cat /sys/class/backlight/*/brightness

The dashboard can control this programmatically based on presence detection.

# 11. Final Checklist

Before rebooting into kiosk mode, verify everything:

## Services Running

sudo systemctl status klair-dashboard
sudo systemctl status klair-hardware

Both should show 'active (running)'.

## Hardware Test

- mmWave sensor detects presence (test script from hardware guide)

- NeoPixel LEDs light up (test script from hardware guide)

- Touch screen responds to input

## Network & APIs

- Pi connected to WiFi

- Can reach N100 server (ping 100.103.162.79)

- Weather API returns data

- RSS feeds loading

## Final Reboot

sudo reboot

The Pi should boot directly into the dashboard in full-screen kiosk mode.

✓ Success! If everything is working, your dashboard is ready. The Pi will automatically start the dashboard on every boot.

# Appendix A: Useful Commands

## Service Management

| Command | Purpose |
| --- | --- |
| sudo systemctl restart klair-dashboard | Restart dashboard server |
| sudo systemctl stop klair-dashboard | Stop dashboard server |
| journalctl -u klair-dashboard -f | View dashboard logs (live) |
| journalctl -u klair-hardware -f | View hardware logs (live) |

## Exit Kiosk Mode

To exit kiosk mode and access the desktop:

- SSH into the Pi from another computer

- Run: pkill chromium

- Or press Alt+F4 on a connected keyboard

## Update Dashboard

cd ~/projects/klair-dashboard
git pull
npm install
npm run build
sudo systemctl restart klair-dashboard

# Appendix B: Troubleshooting

## Dashboard Won't Start

- Check logs: journalctl -u klair-dashboard -n 50

- Verify Node.js: node --version

- Check if port 3000 is in use: sudo lsof -i :3000

## Can't SSH After Kiosk Setup

- SSH should still work—the kiosk only affects the display

- Try: ssh parminder@klair-dashboard.local

- If that fails, connect a keyboard and press Ctrl+Alt+F2 for a terminal

## Screen Goes Blank

- Screen saver may have activated despite settings

- SSH in and run: DISPLAY=:0 xset s off -dpms

- Check lightdm config was saved correctly

## WiFi Disconnects

- Disable WiFi power management:

sudo nano /etc/NetworkManager/conf.d/default-wifi-powersave-on.conf
# Set wifi.powersave = 2 (disable)

## Out of Memory

- Chromium can be memory-hungry. Add swap:

sudo dphys-swapfile swapoff
sudo nano /etc/dphys-swapfile
# Set CONF_SWAPSIZE=1024
sudo dphys-swapfile setup
sudo dphys-swapfile swapon

— End of Software Setup Guide —